GT SYSTEM 1175 WHISPERSLIDE TELESCOPIC AUTOMATIC ENTRANCE SYSTEMS

SUGGESTED ARCHITECTURAL SPECIFICATIONS

SECTION 8

# Part 1-GENERAL

1.01 DESCRIPTION

A.	Furnish and install automatic equipment as indicated on drawings and specifications.

B.	Related work specified elsewhere.

(See note to Specifier*)

1.	Electrical Supply:

Section________

1.02 QUALITY ASSURANCE

A.	Manufacturer’s Qualifications:

Products specified shall be represented by a factory authorized and trained distributor.  Distributor shall maintain a parts inventory and trained service personnel capable of providing service.

B.	All automatic equipment to comply with ANSI A156.10.

C.	Gyro Tech equipment as manufactured by NABCO ENTRANCES, INC. has been specified and shall be quoted as a base bid.  Other systems can be quoted along with information specifically detailing the differences from the following specification.

1.03 SUBMITTALS

A.	Shop drawings showing complete elevations, details and methods of anchorage to location; installation of hardware; size, shape, joints and connections; and details of joining with other construction.

B.	Templates and diagrams and/or shop drawings as needed shall be furnished to fabricators and installers of related work for coordination of sliding door system with concrete work, electrical work, and other related work.

C.	A copy of appropriate manual shall be provided to owner’s representative upon completion of installation.

1.04 WARRANTY

A.	Warranty power operators, controls and labor provided by automatic sliding door equipment installer against defects in material and workmanship, at no cost to owner, for a period of one year from date of substantial completion. Provide warranty to owner after completion of installation.

1.06 COMPLIANCE

A.	A completed American Association of Automatic Door Manufacturer (AAADM) compliance form shall be submitted as proof of compliance with ANSI A156.10 Standard for power operated pedestrian doors.  Door(s) shall be inspected and form shall be signed by an AAADM certified inspector prior to placing door(s) in operation.

## Part 2-PRODUCTS

2.01 APPROVED MANUFACTURER

A.	Automatic equipment and controls shall be manufactured by:

NABCO ENTRANCES INC.

S82 W18717 Gemini Drive

Muskego, WI   53150

Phone: (877) 622-2694

Fax: (888) 679-3319

2.02 AUTOMATIC GT 1175 WHISPERSLIDE TELESCOPIC SYSTEM

A.	GT Model #1175 Telescopic Whisperslide as indicated on door schedule and details.

B.	Mode of operation:  an electro-mechanical operator with a microcomputer control system shall drive Sliding door.  The door will be pulled from closed to open and open to close position stopping the door in both directions by electrically reducing the voltage, stalling door against mechanical stop.  Opening, closing speeds and hold open time shall be adjustable.  A reinforced timing belt shall be used to convert rotating motion from the operator sprocket into horizontal motion of the door

C.	Components:

1.	Aluminum doors, sidelites, operator housing and frame.

2. 	Rollers-support, anti-riser and guide.

3.	Door carrier hanger assembly, breakaway latch, limiting arm and door lock.

4.	Air infiltration and intrusion protection equipment.

5.	Gyro Tech Whisperslide power close operator with micro-computer control.

1a) Door panel(s) and sidelite(s) panel shall be factory assembled with 3/8"-16 threaded

tie rod spanning full length of top and bottom rails.  Snap-in glass stop with integral extruded

vinyl standoff to accommodate glass flexing.  A horizontal muntin bar to provide glass protection.

1b) Operator housing section shall be five piece construction 8-3/4” (222mm) by 7-1/” (191mm) extrusion with enclosed end caps.  All header sections shall have a minimum thickness of 1/8" (3mm) and shall be fabricated of 6063-T5 aluminum alloys.

1c) Sidelite configuration shall be fixed or full breakaway.

1d) Finish:  Aluminum shall have a standard finish of AA-M12-C22-A31 (204R1, clear) or AA-M12-C22-A44 (dark bronze).  Black and special finishes available upon request.

1e) Vertical jambs shall be of 2”51mm) by 7”(178mm) extruded aluminum tubes.

2a) The door assembly shall ride on two 1-13/32" (36mm) dia. steel, urethane coated support rollers incorporating lubricated sealed ball bearings rated at 250 lbs. each.  The door shall be held on the track by means of two 1-13/32" (36mm) anti-riser rollers.  Lateral adjustment of the door assembly shall provide positive sealing at door edges.  Door height shall be adjustable by 7/16" (11mm).

2b) Fixed Sidelite - Each door shall include one guide assembly incorporating double rollers with sleeve bushings.  Guide assembly shall be attached to the door with 10 gage (3mm) thick formed guide bracket.  All steel brackets and fittings shall be plated for corrosion resistance.

2c) Full Open Units - Each door shall include one guide assembly incorporating one roller and guide piston riding in a surface or recessed floor track assembly.

3a) Entrance systems shall have door panels attached to a door carrier hanger assembly by means of an adjustable support rod pivot assembly and corrosion resistant adjustable breakaway release latch holding panel in the closed position under normal automatic operation.  The support rod pivot assembly allows the door panel to be broken outward at any point in the door’s opening or closing cycle allowing for safe emergency egress in compliance with NFPA 101.  The door panel in the breakout mode disconnects the power to the control circuit inhibiting automatic door operation.  The control circuit shall be resettable by re-engaging the door panel with the door carrier hanger assembly.  Breakaway pressure shall be field adjustable (5-50 lbs.) to meet local building code requirements but will be factory set at 50 lbs.  maximum.

3b) Door assemblies shall have a limiting arm to control the door as it swings in the direction of egress.

3c) The lead active door will incorporate a two point lock securing the lead stiles and door carrier hanger assembly.  In the case of a single slider, securing at the jamb and door carrier hanger assembly.  The lock assembly will incorporate a key cylinder on the exterior and a thumb turn on the interior in accordance with NFPA 101.

4) Double pile weatherstripping on the lead edge of the sliding door(s) .36" thick (9mm) including the area of the lock.

4a) 11/16" (17mm) wide nylon brush weatherstripping on the vertical stile of both the sliding door panel(s) and sidelite(s) panels

4b) 11/16" (17mm) wide nylon brush weatherstripping mounted on door bottom.

5) GyroTech Whisperslide Telescopic Power Operator: Completely assembled and sealed unit which shall include gear-driven transmission, and bearings, all located in cast aluminum housing and filled with special lubricant for extreme temperature conditions. Attached to transmission system shall be a  DC shunt-wound permanent magnet motor with sealed ball bearings.   1/10 HP motor shall operate from 115-volt supply and require less than 5 amps at full stall.  To facilitate service and continuing operations, the motor supplied with the slider shall also be able to operate other doors supplied for the project including swing or folding types

5a) Power Operator Control: Shall be a microprocessor unit. To facilitate service and continuing operations, the Control supplied with the slider shall also be able to operate other doors supplied for the project including swing or folding types. The microprocessor control shall allow the opening speed, closing speed, back check speed and latch check speed each to be adjusted separately and independently from each other to meet specific site conditions.  The doors shall be set to be held closed with the motor.  The control system shall also be set up to provide a count of the number of operations of the doors for maintenance purposes.  Adjustable opening and closing speeds shall be set in accordance with ANSI 156.10.  All adjustments shall be specific and reproducible.  Settings with rotary switches are not allowed.

2.03 SENSOR DEVICES

A.	Acusensor: Manufactured by NABCO ENTRANCES, INC.

Sensors for door activation and threshold sensing shall provide a rectangular shaped pattern with a sensing area next to the door system.  To provide optimum coverage to meet specific site conditions the sensing pattern shall be adjustable both in width and depth of coverage while remaining at a full power setting.  Units shall be supplied and installed on both sides of the operator housing to activate doors for single or two-way traffic.  Units shall be sealed for protection against dust, moisture and the elements.

B.	Acumotion:  Manufactured by NABCO ENTRANCES, INC.

Sensors for door activation and threshold sensing shall provide a rectangular shaped pattern with a sensing area next to the door system.  To provide optimum coverage to meet specific site conditions the sensing pattern shall be adjustable both in width and depth of coverage while remaining at a full power setting.  Acumotion uses two technologies for activation and presence sensing.  The activation is achieved by Doppler microwave for long range sensing.  Presence sensing is achieved by active-infrared.  Units shall be supplied and installed on one or both sides of the operator housing to activate doors for single or two-way traffic.  Units shall be sealed for protection against dust, moisture and the elements.

C.	Specification options for consideration:

1.	For others see product catalog.

### PART 3- EXECUTION

3.01 INSTALLATION

A.	Automatic door equipment shall be installed by or under the direction of factory-trained installers in compliance with manufacturer’s recommendations and approved shop drawings.

3.02 CLEANING AND PROTECTION

A.	After installation, clean framing members as recommended by the manufacturer.  Aluminum surfaces in contact with masonry, concrete or steel shall be protected from contact by use of neoprene gaskets, where indicated, or a coat of bituminous paint to prevent galvanic or corrosive action.  Advise general contractor to protect unit from damage during subsequent construction activities.

*	COVER NOTE TO SPECIFICATION WRITER

Indicate under appropriate Section the following work by others:

ELECTRICAL INSTALLER shall furnish and install all conduit and electrical wiring for activating devices and door operators.  A minimum of 5 amperes, 115 volts, A/C, 1-phase circuit shall be furnished for each door operator, terminate and connect to operator control panel, in operator housing.

CONCRETE INSTALLER shall prepare floor at location of automatic entrance system to accommodate the surface or recessed electric switch mats indicated on drawings.